s

SPILL RESPONSE PROCEDURE 1 BE SAFE • Identify spilled substance and any dangers. • Emergency services required? Dial 111. • Put on appropriate PPE and proceed if safe to do so. 2 STOP THE SOURCE • Turn off taps, pumps, and valves. • Stand container upright or roll it over. • Plug leak or place a drip tray under it until fixed; empty trays before they overflow. 3 PROTECT STORMWATER • Confine a liquid spill with sandbags or booms. • Cover a dry spill to protect it from wind and rain if required. • Block off access to stormwater drains with drain covers. 4 NOTIFY • Tell your supervisor. • Dial ECan Pollution Hotline (0800 765 588) if the spill reaches land, drains, or waterways. 5 CLEAN UP • Neutralise hazardous substances. • Transfer liquid to a safe container or absorb it with a compatible material. • Sweep up a dry spill and spent absorbents promptly. • Clean up confined area and do not track a spill to other areas. 6 DISPOSE RESPONSIBLY • Never hose a spill away; always prevent washwater and sweepings from entering stormwater drains. • Call a waste disposal contractor to take away contaminated materials. 7 RESTOCK AND REVIEW • Replace materials and PPE at spill station. • Learn how and why the spill happened and what corrective actions are needed.